Lab report 05

Adder and Subtractor

_____________________________

Group No 05

202L-Digital Logic Design Lab

Department of Computer System Engineering

University of Engineering and Technology Peshawar

Submitted to: Engr. Rehmat Ullah

Submitted by

Muhammad Zaid      (1991)

Muhammad Saad     (1997)

Muhsin Shah    (2018)

DCSE, Batch 23, Section “B”

Adder and Subtractor

Objective:

- To design half adder, half subtractor, full adder and full subtractor and verify their truth table

Apparatus:

- Logic gates

- AND gate (IC 7408)

- OR gate (IC 7432)

- XOR gate (IC 7486)

- NOT gate (IC 7404)

2.	LEDs

3. 	DIP Switch

4.	520 Ω / 1k Ω resistors

Theory:

An adder or subtractor adds or subtracts two binary inputs of one bit. Half adder and half subtractor can only add two one bit binary numbers and in output it has also two numbers one is the sum and other is the carry or borrow. But as it can only operate on two inputs and there room for carry or borrow from the previous stage makes it limiting and that is why it is called half adder and subtractor. A full adder on the other hand can also take another input of carry or borrow from previous stage and therefore can add any two numbers. We can also chain multiple full adders which is called ripple carry adder.

Circuit Diagrams and truth tables

Half Adder:

It is a one bit adder which add two one bit numbers and then gives the result as sum and carry.

Fig. 01  Half adder circuit diagram

S(A, B)= A⊕B

C(A, B) = A.B

| A | B | S(A, B) | C(A, B) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Half Adder

Half Subtractor:

It also adds two one bit numbers and gives the result as difference and borrow. The connections are according to circuit diagrams.

Fig 02. Half subtractor

D(A, B)= A⊕B

Bo(A, B) = A’ . B

| A | B | D(A, B) | Bo(A, B) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Half Subtractor

Full Adder:

Full adder takes two inputs and one comes Cin comes from the previous stage.

Fig 03. Full Adder

S(A, B, Cin) = A⊕B⊕Cin

Cout = A⊕B.Cin + A.B

| A | B | Cin | S(A, B, Cin) | Cout(A, B. Cin) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Full Adder

Full Subtractor:

Full subtractor like full adder has two input and third input called Borrow (Bin ). The result is a difference and borrow.

Fig 04. Full Subtractor

S(A, B, Cin) = A⊕B⊕Bin

Bout = (A⊕B)’ .Bin + A’ . B

| A | B | Bin | D(A, B, Bin) | Bout(A, B. Bin) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Full Subtractor

Experiment Pictures