Lab report 05

BCD to 7-Segment Decoder

_____________________

308L-Digital Systems Design LAB

Department of Computer System Engineering

University of Engineering and Technology Peshawar

Submitted to: Engr. Shahzada Fahim Jan

Submitted by: Muhammad Saad

Reg ID: 21PWCSE1997

DCSE, Batch 23, Section “B”

Date: SUN, 31st March, 2024

Lab 05

BCD to 7-Segment Decoder

Objective:

- To implement a BCD to Seven Segment Decoder on Spartan 6 board

Components needed for this lab:

- Xilinx ISE

- Spartan 6 Board

BCD to 7-Segment Display:

In this lab, we will take a BCD input from the user and display the corresponding number on a seven-segment display. The diagram shows the 7-bit code for displaying the digit ‘2.’ tTe enable signal must be low to activate a specific seven-segment display. The Spartan 6 board has four such displays, and later labs will cover time multiplexing techniques to control all four displays using shared input signals (A, B, C, D, E, F, G, Dp).

Fig 01: BCD pin and display configuration

Tasks

module disp(O, dp, A, en );

output reg [6:0]O;

output dp;

input [3:0]A;

input en;

assign dp = 0;

always@(*) begin

if(A==4'b0000)

O <= 7'b1000000;//0

else if(A==4'b0001)

O <= 7'b1111001;//1

else if(A==4'b0010)

O <= 7'b0100100;//2

else if(A==4'b0011)

O <= 7'b0110000;//3

else if(A==4'b0100)

O <= 7'b0011001;//4

else if(A==4'b0101)

O <= 7'b0010010;//5

else if(A==4'b0110)

O <= 7'b0000010;//6

else if(A==4'b0111)

O <= 7'b1111000;//7

else if(A==4'b1000)

O <= 7'b0000000;//8

else if(A==4'b1001)

O <= 7'b0010000;//9

end

endmodule

1: Implement buffer ON the Kit and attach the snapshot.

Verilog Code for 7-Segment BCD

NET "A[3]" PULLUP;

NET "A[2]" PULLUP;

NET "A[1]" PULLUP;

NET "A[0]" PULLUP;

NET "O[6]" SLEW = FAST;

NET "O[5]" SLEW = FAST;

NET "O[4]" SLEW = FAST;

NET "O[3]" SLEW = FAST;

NET "O[2]" SLEW = FAST;

NET "O[1]" SLEW = FAST;

NET "O[0]" SLEW = FAST;

# PlanAhead Generated physical constraints

NET "A[3]" LOC = F17;

NET "A[2]" LOC = F18;

NET "A[1]" LOC = E16;

NET "A[0]" LOC = E18;

NET "O[0]" LOC = A3;

NET "O[1]" LOC = B4;

NET "O[2]" LOC = A4;

NET "O[3]" LOC = C4;

NET "O[4]" LOC = C5;

NET "O[5]" LOC = D6;

NET "O[6]" LOC = C6;

NET "dp" LOC = B3;

NET "en" LOC = C17;

UCF File Code

Fig 02: First we turn on ‘Create Binary Configuration File’

Fig 03: Choosing input and output ports for spartan 6 board

Fig 04: Programming Spartan 6 board with .bin file

Fig 05: BCD Displaying 0

Fig 06: BCD displaying 1

Fig 07: BCD displaying 2

Fig 08: BCD displaying 3

Fig 09: BCD goes upto 9

___________________________________________________________________________________________________________

2: Connect the output of your lab 02 (4 bit adder) to the seven segment display. Note that number above 1001 are not valid

BCD numbers. In this situation keep the seven segment display off and just the dp on.

module disp(O, dp, A, B, en );

output reg [6:0]O;

output reg dp;

input [3:0]A, B;

input en;

wire [3:0]sum;

assign sum = A+B;

always@(*) begin

if(sum==4'b0000)

O <= 7'b1000000;//0

else if(sum==4'b0001)

O <= 7'b1111001;//1

else if(sum==4'b0010)

O <= 7'b0100100;//2

else if(sum==4'b0011)

O <= 7'b0110000;//3

else if(sum==4'b0100)

O <= 7'b0011001;//4

else if(sum==4'b0101)

O <= 7'b0010010;//5

else if(sum==4'b0110)

O <= 7'b0000010;//6

else if(sum==4'b0111)

O <= 7'b1111000;//7

else if(sum==4'b1000)

O <= 7'b0000000;//8

else if(sum==4'b1001)

O <= 7'b0010000;//9

else if(sum>4'd9)

begin

dp <= 0;

O <= 0;

end

end

endmodule

Verilog Code for 7-Segment BCD

NET "A[3]" PULLUP;

NET "A[2]" PULLUP;

NET "A[1]" PULLUP;

NET "A[0]" PULLUP;

NET "O[6]" SLEW = FAST;

NET "O[5]" SLEW = FAST;

NET "O[4]" SLEW = FAST;

NET "O[3]" SLEW = FAST;

NET "O[2]" SLEW = FAST;

NET "O[1]" SLEW = FAST;

NET "O[0]" SLEW = FAST;

NET "A[3]" LOC = F17;

NET "A[2]" LOC = F18;

NET "A[1]" LOC = E16;

NET "A[0]" LOC = E18;

NET "O[0]" LOC = A3;

NET "O[1]" LOC = B4;

NET "O[2]" LOC = A4;

NET "O[3]" LOC = C4;

NET "O[4]" LOC = C5;

NET "O[5]" LOC = D6;

NET "O[6]" LOC = C6;

NET "dp" LOC = B3;

NET "en" LOC = V16;

NET "B[3]" LOC = D18;

NET "B[2]" LOC = D17;

NET "B[1]" LOC = C18;

NET "B[0]" LOC = C17;

# PlanAhead Generated IO constraints

NET "B[3]" PULLUP;

NET "B[2]" PULLUP;

NET "B[1]" PULLUP;

NET "B[0]" PULLUP;

UCF File Code

Fig 10: Choosing input and output ports for spartan 6 board

Fig 11: Programming Spartan 6 board with .bin file

Fig 12: BCD Displaying 0

Fig 13: BCD displaying 1

Fig 14: BCD displaying 2

Fig 15: BCD displaying 3

Fig 16: BCD goes upto 9

___________________________________________________________________________________________________________

Conclusion:

In this Lab we learned how to program an FPGA (spartan 6) for showing 1 to 9 numbers on 7-segment display in first task and in the second we created a 4-bit adder and showed the result on 7-segment display.

____________________________________________________________________________________________________________________________