MICROCONTROLLER BASED SYSTEM DESIGN

DEPAERTMENT OF COMPUTER SYSTEM ENGINEERING

UNIVERSITY OF ENGINEERING AND TECHNOLOGY PESHAWAR

NAME: Muhammad Ilyas

REG ID; 21PWCSE2055

SECTION: B

Lab No 9

Design of a Light switch

Objective: Build a light switch controller such that when the push button is pressed the light if “off” turns “on” and if “on” turns “off”.

Block Diagram:The top level block diagram is shown in the following figure. The second diagram is a more detailed view of the top level diagram. Synchronizer circuit is used to avoid the metastability problem which arises when synchronous digital system are fed with an asynchronous input. The level to pulse converter is there to convert level input from a push button into a pulse that is high for only one clock cycle. The switch button FSM is actually implementing the state machine for the requirement i-e when the push button is pressed the light if “off” turns “on” and if “on” turns “off”.

I/O connection: Connect the button input to a push button on the S3BOARD and light output to LED.

Synchronizer Circuit:

Level to Pulse FSM:

Switch Button FSM:

Task 01: Create an FSM having two state. LED will be on in one state and off in other state.

//clock_divider

module clock_divider(

input clk_in,

output reg clk_out

);

reg [27:0] counter = 28'd0;

parameter divisor = 28'd1000000;

always @(posedge clk_in) begin

counter <= counter + 28'd1;

if (counter >= (divisor - 1))

counter <= 28'd0;

clk_out <=(counter < divisor/2)?1'b1:1'b0;

end

endmodule

Clock Divider Module

// Synchronizer

module synchronizer(input clk, input btn, input rst, output synch_btn);

wire Q1;

D_FF df1(Q1, btn, clk, rst);

D_FF df2(synch_btn, Q1, clk, rst);

endmodule

// D_FlipFlop

module D_FF(Q, D, clk, rst);

input D; // Data input

input clk; // Clock input

input rst;

output reg Q; // Output Q

always @(posedge clk or negedge clk) begin

if (rst)

Q = 1'b0;

else

Q = D;

end

endmodule

Synchronizer and D-FlipFlops

module fsm(input btn0,

input clk,

input RST_BTN,

output reg led);

reg [2:0] presentState, nextState;

parameter s0 = 3'b0, s1 = 3'b1;

always @(posedge clk)

presentState <= nextState;

always @(nextState or btn1 or RST_BTN) begin

if (RST_BTN)

nextState <= s0;

else

case (presentState)

s0: nextState <= btn0 ? s1 : s0;

s1: nextState <= btn0 ? s0 : s1;

endcase

end

assign led = (presentState == s1);

endmodule

FSM Module

module level_to_pulse(

input synch_input,

input clk,

input rst,

output pulse

);

wire Q;

D_FF df(Q, synch_input, clk, rst);

and a(pulse ,~Q, synch_input);

endmodule

Level to Pulse Converter Module

module btn_module(

input btn,

input CLK,

input RST,

output pulse

);

wire synch_btn;

synchronizer s1(CLK, btn, RST, synch_btn);

level_to_pulse lp1(synch_btn, CLK, RST, pulse);

endmodule

Module taking input from from Push button

module Top( CLK, RST, RST_BTN, , BTN, LED, SEVENSEG);

input CLK, RST, BTN,  RST_BTN;

output LED;

wire SLOW_CLOCK;

wire pulse0, pulse1, RST_Pulse;

wire [3:0]bcd;

clock_divider divider(CLK, SLOW_CLOCK);

fsm my_fsm( pulse1, SLOW_CLOCK, RST_Pulse, LED, bcd);

btn_module b0(BTN, SLOW_CLOCK, RST,  pulse1);

btn_module b2(RST_BTN, SLOW_CLOCK, RST,  RST_Pulse);

endmodule

Top Level Module

----------------------------------------| Output |------------------------------------------

______________________________________________________________